ZWS Data Collection

Observation report

Site: Stewart Milne: SM Homes Project.  Coulter's Brae, Carmunnock

Date: 10/02/2020

By: Naser Mohamed

Introduction:

Naser arrived on site on Monday the 10th of February at 08:00 for site induction and initial assessment.

This site uses a 40 yards mixed waste skip; it was not possible to facilitate an 8 yard skip therefore the equivalent of 8 yard skip was calculated.

The site usually uses mini skips (1200 Litres) to transport waste from homes to 40 yards mixed waste skip.

Approximately 7 of these mini skips are equivalent of 8 yard skip, based on the following calculations;

8 yard skip is 6.12 cubic meters,

1 mini skip is 1.2 cubic meter (subtracting 20% compression effect equals 0.96 cubic meters), Therefore a minimum of 6.3 it skips will be required, it was agreed to collect data from 7 of these mini skips.

Waste management on site:

This site segregate the waste generated onsite into the following skips,

- 40 yards wood/timber waste skip

- 14 yards gyprock wastes skip.

The usual practice is to provide each house (block) with two mini skips, one for mixed/general waste and the other is for the specific waste generated by the trade working on the block (i.e. wood works, plastering, roofing, pluming, etc.) brickwork waste usually used at the location to support ground stability for driveways. When a mini skip is filled up; it is transferred to the skips yard and emptied as appropriate.

Data entry:

In the data collection sheet of this project (see enclosed), the change of colures of ‘Time of day’ column refers to the separation between mini skip to the next one

The Stewart Milne team:

3 of Stewart Milne staff were directly involved in this field study.

Days on site:

| day | date | From - to | Note |
| --- | --- | --- | --- |
| Monday | 10 Feb 20 | 08:00 - 10:30 | Stormy weather, minimum work onsite |
| Wednesday | 12 Feb 20 | 09:00 - 16:30 |  |
| Thursday | 13Feb 20 | 09:00 - 16:30 | Study completed |

Site layout and waste travel distance:

The site is very large with contractors working all around the area, most of the time it was not possible to determine where exactly the waste come from as contractors keep moving around site and towards the end of the shift all transported to the skips. However, when items identified the approximate location noted on the map. All welfare waste comes from the site offices, canteens, and toilets (see ‘site offices’ location on the map below); a full site layout is enclosed.